# Digitization Standards & Specifications

University of Texas Libraries Digitization Services 02/02/2017

Subject specialists and other Libraries staff responsible for collection decisions determine what University of Texas Libraries materials are desirable as digitization projects. With the exception of day-to-day and one-off media reformatting, prioritization of digital projects is assessed by the Libraries Digital Projects Oversight Committee (DPOC) or its replacement.  Digitization is a single part of the over-all Digital Projects workflow and a crucial aspect of collections care through increasing access to and reducing physical handling of fragile analog materials.

UT Libraries Digitization Services adheres to international standards and best practices for digitization, including the Federal Digitization Guidelines Initiative, and produces a range of quality archival and access media derivatives, useful to our faculty, students, staff, visiting researchers and the public. Although DPOC may determine the ideal capture and access specifications for specific Digital Projects, Digitization Services has established and adheres to the following Digitization Standards & Specifications across workflows.

| Media Type: Film | Archival Master: DV25 codec in AVI wrapper | Production Master: N/A | Access Copy: QuickTime H.264 MPEG4 encoding as .mp4 file | DVW Hardware / Software |
| --- | --- | --- | --- | --- |
| - 8mm - 16mm | - Size: 720x480 - 5:1 compression - 4:1:1 color space - 25 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency - Average file size: 13 GB/hr. | None | - Format: MPEG4 - Size: 320x240 - Frame rate: 30.07 - Hinting: enabled - Audio: AAC, 44.1kHz stereo, 16-bit - Average File Size: 162 MB/hr. | Canopus ADVC110 using Sony Vegas (Outsourced) |
| Media Type: Analog Video | Archival Master: Uncompressed 10-bit video in QuickTime MOV wrapper (as appropriate) | Production Master: Apple Pro-Res 422 in QuickTime MOV wrapper | Access Copy: QuickTime H.264 MPEG4 encoding as .mp4 file | DVW Hardware / Software |
| - UMatic - BetaMax - VHS - Video 8mm - Hi8 | - NTSC: - Size: 720x486 - No compression - 4:2:2 color space - 29.97 fps = 27 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency - Average file size: 94 GB/hr. - PAL: - Size: 720x576 - No compression - 4:2:2 color space - 25fps = 26 Mbps data rate - Average file size: 94 GB/hr. | - Size: same as source - 3:1 compression - 4:2:2 color space - 63 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency - Average file size: 18 GB/hr. | - Format: MPEG4 - Size: 320x240 - Frame rate: 30.07 - Hinting: enabled - Audio: AAC, 44.1kHz stereo, 16-bit - Average File Size: 162 MB/hr. | AJA IO LA or BlackMagic UltraStudio 4k (uncompressed workflows) Canopus ADVC300 (compressed workflows) |
| Media Type: Digital Video | Archival Master: NTSC DV codec in QuickTime MOV wrapper | Production Master: NA | Access Copy: QuickTime H.264 MPEG4 encoding as .mp4 file | DVW Hardware / Software |
| - Digital 8 - MiniDV | - Size: 720x480 - 5:1 compression - 4:1:1 color space - 25 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency - Average file size: 11 GB/hr. | None | - Format: MPEG4 - Size: 320x240 - Frame rate: 30.07 - Hinting: enabled - Audio: AAC, 44.1kHz stereo, 16-bit - Average File Size: 162 MB/hr. | Canopus ADVC300 or FireWire direct using Final Cut Pro |
| Media Type: File-based Video | Archival Master: ISO image / Disk image | Production Master: NA | Access Copy: QuickTime H.264 MPEG4 encoding as .mp4 file | DVW Hardware / Software |
| - DVD - Vendor supplied video file - video file on external media / drive | (documentation available separately) | None | - Format: MPEG4 - Size: 320x240 - Frame rate: 30.07 - Hinting: enabled - Audio: AAC, 44.1kHz stereo, 16-bit - Average File Size: 162 MB/hr. | (documentation available separately) |
| Media Type: Audio | Archival Master: Uncompressed Broadcast Wave Format audio in WAV file format wrapper | Production Master: Uncompressed Broadcast Wave Format audio in WAV file format wrapper | Access Copy: MPEG-2 Audio Layer III in MPEG-3 wrapper | DAW Hardware / Software |

| - 1/4 in. open reel - 78/45/331/3 rpm grooved disc - Micro/mini and Standard cassette - DAT tape - PCM audio on VHS - 8-track cassette - Audio CD | - 96 kHz, 24 bit PCM digital audio. - Average file size: 2GB/hr. | - 48 kHz, 24 bit PCM digital audio. - Average file size: 500MB/hr. | - 44.1 kHz, 16 bit 128 kB/s at best encoding. - Average file size: 60 MB/hr. | DigiDesign Rack 003 using Pro Tools |
| --- | --- | --- | --- | --- |