# Digitization Standards & Specifications

University of Texas Libraries Digital Stewardship – Digitization Services

Staff from across University of Texas Libraries prioritize digitization and digital preservation of UTL collection materials in partnership with Digital Stewardship. Day-to-day prioritization of digitization is assessed by Digital Stewardship.

UT Libraries Digitization Services adheres to international standards and best practices for digitization and produces a range of quality archival and access media derivatives useful to our faculty, students, staff, visiting researchers and the public. Although individual collection curators may determine the ideal access specifications for specific digital projects, Digitization Services has established and adheres to Digitization Standards & Specifications across workflows as outlined in this document.

# File Format Selection and Production Specifications:

File format selection and production specifications have a direct effect on the performance of the digital object as well as implications for long term management. Formats and specifications utilized by UTL Digital Stewardship adhere to Federal Digitization Guidelines Initiative (FADGI) 4 Star recommendations for creating and managing raster image files; and recommendations from the International Association of Sound and Audiovisual Archives (IASA) for the production and preservation of audiovisual and sound objects whenever feasible.

UTL Digital Stewardship acknowledges the emerging nature of audiovisual digitization, reformatting, and encoding specifications. With this in mind, we currently employ marketplace wrappers (i.e. QuickTime/OpenDML AVI) which are recognized by both FADGI and IASA as acceptable in terms of sustainability, quality, and functionality factors because these formats maintain a relatively high level of adoption across industries, making them easier to manage in existing production and QC workflow environments.  However, as preferred wrappers (i.e. MXF and Matroska) gain wider implementation, UTL will continue to reassess moving to these target formats in the future.

For additional information on file formats and format selection please reference The Library of Congress Sustainability of Digital Formats web site: http://www.digitalpreservation.gov/formats/index.shtml

Master Files:

Archival master files represent the best copy produced, in a highly sustainable format. These files should be minimally adjusted to produce use-neutral copies of the original content. For raster image files, it is recommended to convert proprietary or camera raw images to TIFF. Raw images are not recommended according to FADGI guidelines.

Production master files are produced by processing the content in one or more archival master files, resulting in a new file or files with levels of quality that rival those of the archival master. For example, an image of a large map may be produced by stitching together a set of image tiles, each representing a portion of the original paper item. Other processes that may be applied include aesthetic or other technical corrections to the archival master file. For example, analog audio recorded at an unsupported IPS setting may require time and pitch adjustment to render the output intelligible.

Both archival master and production master files are intended for long term maintenance and may not function well in access environments.

Access Files:

Access files use master files as a data source and are output to optimize a particular use. Typical uses (each of which may require a different optimization) include the provision of end-user access; high quality reproduction; or the creation of textual representations via OCR. Access files may employ lossy compression and selected formats may become obsolete in a relatively short time. Therefore, these files are generally not intended for long term maintenance.

File Compression:

Compression may be appropriate for both master and derivative files. Significant benefits can result from the appropriate use of file compression. Lossless compression such as JPEG 2000 and FFV1 for master files are approved for some uses. Compression techniques using patented or proprietary programs are avoided due to long term sustainability concerns and lossy compression is generally not employed by UTL Digital Stewardship for master files.

# File Management

Master Files:

Master files of all types have permanent value to our designated community and are subject to preservation-oriented data management actions. Therefore, master files are stored in an environment meant to mitigate loss of essential features over time (i.e. tape or offline cloud storage).

Archival master files are appended with the suffix “am” and stored within the preservation package as “files” while production master files are appended with the suffix “pm” and stored within the preservation package as “derivatives.”

Access Files:

Access files are frequently recalled by end-users and are typically stored in systems that see repeated read and write executions, introducing the potential for compromised file integrity over time e.g. the UTL DAMS and Texas ScholarWorks. Therefore, an access file may also be subject to preservation-oriented data management if it is in a sustainable file format and would require substantial Digital Stewardship resources to reproduce.

Access files meeting these requirements are appended with the suffix “ac” and stored within the preservation package as “derivatives,” or they may employ unique access file naming properties and file directory hierarchies, especially those systematically generated using specialized output software (e.g. docWorks). Preservation payloads include descriptions of access file naming output as necessary.

Relationships represented in file-names:

UTL Digital Stewardship considers that simple, unique, meaningless names for file identifiers, coupled with more sophisticated metadata describing relationships across files stored in an external database, is the preferred way forward to link files together. This metadata might include file identifiers and metadata record identifiers and a codified or typed set of relationships that would help define the associations between digital files and between different representations of the same resource. (Relationships between the digital object and the analog source object or the place of the digital object in a larger collection hierarchy would be documented elsewhere in descriptive metadata). Possible relationship types include identification of principal or authoritative version (for master image file); derivation relationships indicating what files come from what files; whether the images were created in-house or come from outside sources; structural relationships (for multi-page or –part objects); sibling relationships (images of the same intellectual resource, but perhaps scanned from different source formats).

One advantage of a non-descriptive file naming convention is that it eliminates non-unique and changeable descriptive data and provides each file with a non-repeating and sustainable identifier in a form that is not content-dependent. This allows much greater flexibility for automated data processing and migration into future systems. Other benefits of a non-descriptive file naming convention include the ability to compensate for multiple object identifiers and the flexibility of an external database, which can accommodate structural metadata including parts and related objects, as well as avoiding any pitfalls associated with legacy file identifiers. UTL Digital Stewardship will provide in-house file and directory naming schema documentation upon request.

# Targets and Color Checkers

The use of targets and color checkers is recommended for digital imaging. UTL Digital Stewardship uses ISA object-level targets as well as GoldenThread/DICE device level targets in all projects.

Placement of Target

Position the target close to, but clearly separated from, the originals being scanned. There should be enough separation to allow easy cropping of the image of the original to remove the target(s) if desired, but not so much separation between the original and target(s) that it dramatically increases the file size.

If it fits, orient the target(s) along the short dimension of originals. That will produce smaller file sizes compared to having the target(s) along the long dimension.

Consider generating publication and access derivatives with the reference target(s) cropped out. This will reduce file size for the access files and present an uncluttered appearance to the images presented.

# Specification Charts: Book, paper, and transparency materials

Bound Volumes: Rare and Special Materials Rare and special bound materials represent various types ranging from illuminated manuscripts, incunabula, works that feature illustrations of special artistic or graphic interest, e.g., intaglios, gravures, or inset photographs; also bound documents with poor legibility or diffuse characters, e.g., carbon copies, Thermofax, etc.

Archival Master File Format: TIFF, JPEG 2000, PDF/A Output details - Resolution: 400 ppi - Bit Depth: 16 - Color Space: Adobe 1998, ProPhoto, ECIRGBv2 - Color: Color Recommended Imaging Technologies - Manually operated planetary book scanners without glass or plastic platens - Digital cameras with book cradles without glass or plastic platens Not Recommended Imaging Technologies - Flatbed scanners - Automated page turning book scanners Note To be FADGI compliant, all imaging performed on special collections materials must be done by personnel with advanced training and experienced in the handling and care of special collections materials. FADGI compliance requires proper staff qualifications in addition to achieving defined performance levels.

Bound Volumes: General Collections Textual and illustrated bound printed matter (books, journals) generally represent clean, high-contrast book pages with clearly legible type, e.g., evenly printed typeset or laser printed pages without background discoloration. Visual-arts elements of limited significance and generally consisting of printed halftones, line art, explanatory tables and drawings, and the like are often included.

Archival Master File Format: TIFF, JPEG 2000, PDF/A Output details - Resolution: 400 ppi - Bit Depth: 8 or 16 - Color Space: SRBG, Adobe 1998, ProPhoto, ECIRGBv2 - Color: Color Recommended Imaging Technologies - Planetary book scanners without glass or plastic platens - Digital cameras Not Recommended Imaging Technologies - Flatbed scanners

Documents (Unbound): Manuscripts and Other Rare and Special Materials Rare and special materials represent manuscripts, illustrations of special artistic or graphic interest; also documents with poor legibility or diffuse characters, e.g., carbon copies, Thermofax, etc.

Archival Master File Format: TIFF, JPEG 2000 Output details - Resolution: 400 ppi - Bit Depth: 16 - Color Space: Adobe 1998, ProPhoto, ECIRGBv2 - Color: Color Recommended Imaging Technologies - Planetary scanners – manually operated - Digital cameras

Oversize Items: Maps, Posters, and Other Materials Materials with Challenging Features That Will Benefit from High Resolution Reproduction

Archival Master File Format: TIFF, JPEG 2000 Output details - Resolution: 400 ppi - Bit Depth: 8 or 16 - Color Space: Adobe 1998, ProPhoto, ECIRGBv2 - Color: Color Recommended Imaging Technologies - Planetary scanners - Digital cameras - Flatbed scanners Notes - For master files, documents should be imaged to include the entire area of the original and a small amount beyond to define the area. - At 4 star, no software de-skew is permitted. Images must be shot to a +/- 1 degree tolerance. Rotation at only 90 degree increments is permitted

Newspapers

Archival Master File Format: TIFF, JPEG 2000, PDF/A Output details - Resolution: 400 ppi - Bit Depth: 8 - Color Space: Grey Gamma 202, SRGB - Color: Color Recommended Imaging Technologies - Planetary scanners - Digital cameras - Flatbed scanners - Sheet fed scanners Notes Newspaper digitization spans the range from extremely fragile documents to modern paper which can be handled easily when new, but ages quickly. Fragile newspapers should only be imaged on scanners that do not stress the paper. Modern newspapers may be suitable for scanning through pass through scanners.

Prints and Photographs Includes photographic prints, graphic-arts prints (intaglio, lithographs, etc.), drawings, some paintings, (e.g., water colors), and some maps.

Archival Master File Format: TIFF Output details - Resolution: 600 ppi - Bit Depth: 16 - Color Space: Adobe 1998, ProPhoto, ECIRGBv2 - Color: Color Recommended Imaging Technologies - Planetary scanners - Digital cameras - Flatbed scanners Not Recommended Imaging Technologies - Drum scanners Notes The use of glass or other materials to hold an image flat during capture is allowed, but only when the original will not be harmed by doing so. Care must be taken to assure that flattening a photograph will not result in emulsion cracking, or the base material being damaged. Tightly curled materials must not be forced to lay flat.

Photographic Transparencies: 35mm to 4”x5” Black-and-white or color (positive) transparencies that can be captured without the need for post-capture assembly of tiles to produce a complete image.

Archival Master File Format: TIFF Output details - Resolution: 4000 ppi - Bit Depth: 16 - Color Space: Grey Ganna 2.2, Adobe 1998, ProPhoto, ECIRGBv2 - Color: Grayscale or Color as appropriate Recommended Imaging Technologies - Film scanners - Planetary scanners - Digital cameras - Flatbed scanners Not Recommended Imaging Technologies - Drum scanners Notes - Appropriate lens selection for film scanning is critical. Lenses designed for general imaging are not appropriate for close focusing film scanning applications. Lenses must be of a flat field design and have the ability to focus all colors at the same plane (APO). - Sufficient illumination intensity must be available to image at the best performance level for the imaging system. The optimum aperture should be determined through testing, and is generally one to two f stops from maximum aperture. Exposure time should be short enough to eliminate the possibility of vibration affecting resolution.

# Specification Charts: Audiovisual and Sound Objects

Film (outsourced) - 8mm - 16mm

Archival Master: Uncompressed or lossless wrapped in QuickTime or AVI Dark storage only Output details Minimum acceptable - Size: 720x480 - 5:1 compression - 4:1:1 color space - 25 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency Average file size: 13 GB/hr.

Analog video - U-matic ¾-inch Videocassettes - ½-inch Analog Consumer and Semi-Professional Videocassettes (eg VHS) - Betacam ½-inch Professional Videocassettes

Archival Master: Uncompressed v210 wrapped in QuickTime Dark storage only Output details NTSC and PAL - Size: 720x486 (PAL: 720x576) - No compression - 4:2:2 color space - 29.97 fps = 27 Mbps data rate (PAL: 25fps = 26 Mbps data rate) - Audio: PCM, 48kHz stereo, 16-bit sampling frequency Average file size: 94 GB/hr.

Digital videotape - DigiBeta - DV family (eg MiniDV)

Archival Master: Lossy, compressed content extracted as data (DV codec) wrapped in QuickTime Dark storage only Output details - Size: 720x480 - 5:1 compression - 4:1:1 color space - 25 Mbps data rate - Audio: PCM, 48kHz stereo, 16-bit sampling frequency Average file size: 11 GB/hr.

File-based Video - Optical media/DVD - Video file on external media/drive

Archival Master: ISO image/disk image Dark storage only Output details available separately

Analog Audio - 78/45/331/3 rpm grooved disc - 1/4 in. open reel tape - Standard cassette tape - Micro/mini cassette tape - 8-track cassette tape - PCM audio on VHS tape - DAT/R-DAT - Audio CD

Archival Master: Uncompressed Broadcast Wave Format audio in WAV file format wrapper Output details - 96 kHz - 24 bit - PCM digital audio Average file size: 2GB/hr. Production Master: Edited or enhanced version of Archival Master as uncompressed Broadcast Wave Format audio in WAV file format wrapper Output details - Same as Archival Master OR - reduced sample/bit rate not less than 48kHz/24bit Average file size: 2GB/hr - 500MB/hr Access: MPEG-2 Audio Layer III in MPEG-3 wrapper Output details - 44.1 kHz - 16 bit Average file size: 128 kB/s at best encoding